| QTC (Quantum Tunnelling composite) What is it? It’s a flexible polymer that contains tiny metal particles. It can come as a sheet, pill or more recently a printable ink. | What does it do? A pressure-sensitive material, it is normally an insulator (does not conduct electricity) but if pressure is applied it becomes a conductor (allows electricity to flow) |
| --- | --- |
| What is it used for? High tech clothing (key pads for MP3 players. Switches used on mobile phones Games- to control how hard you want to jump or run. Speed control on machines. Robotics- touch sensitive fingertips. | Pros QTC can be made very thin- about the thickness of one human hair- this means it’s very sensitive to pressure. It’s very lightweight. |

| SMA (Shape memory alloys) What is it? An alloy (mix of two or more metals) |  | What does it do? When bent out of shape, can be heated above a certain temperature it will return to its original shape. |
| --- | --- | --- |
|  | What is it used for? Spectacle frames- when damaged (say bent) can be heated and return to normal shape. When cool SMA tubes are compact- they can be placed into an artery in the body, when the tube reaches body temperature it will expand allowing blood to flow through the artery. Dental braces- SMA maintain their shape since they are at a constant temperature Dental braces- SMA maintain their shape since they are at a constant temperature. | Pro’s The material can “remember” two distinct shapes, a high temperature shape and a low temperature shape. |
| Polymorph What is it? Small pellets which fuse together when heated to 62˚C. |  | What does it do? It is a tough material yet becomes easily mouldable at 62°C. Unused pellets can be re-heated and re-used. |
|  |  | Pro’s Once cooled, polymorph has the advantage of hardening like a plastic, whereas Plasticine remains soft. Polymorph can be re-heated to create new shapes, allowing the user to form the plastic by hand into unique shapes. |
| What is it used for? Create prototypes and solve manufacturing problems. Vacuum forming moulds Manufacturing awkward shapes such as ergonomic handles in prototyping. Orthopaedic aids |  |  |
| Biometrics What is it? Used in computer science as a form of identification and access control. |  | What does it do? Biometrics identifies an individual from a physical characteristic, for example fingerprint, face recognition, iris recognition. |
|  |  | Pro’s Since biometric identifiers are unique to individuals, they are more reliable. |
| What is it used for? - Airport security. Iris recognition has been used in a number of large airports for several years. . - Schools-fingerprint for school dinners - Mobile phone- I phone fingerprint recognition. - Voice recognition in cars. |  |  |
| Microencapsulation What is it? Tiny microscopic bubbles filled with a substance (for example anti-bacterial) that are sprayed onto the material. |  | What does it do? When they come into contact with skin, they burst and slowly release the substance. |
| What are they used for? Anti-bacterial socks, underwear and bandages to name a few. |  | Pro’s They can be filled with a number of different substances For example: - Fragrances (perfume) - Moisturisers - Insect replant - Vitamins - Anti-bacteria substance. |

| Photochromic dyes What is it? It is a smart dye that can be added to plastics and fabrics. | What does it do? Photo chromatic dyes change colour in response to UV light/sunlight. |
| --- | --- |
| What are they used for? Decoration purposes on clothes. Exposure to sunlight causes the lens of the glasses to darken to protect the eye. | Pro’s Glasses- They provide greater comfort, since they reduce eyestrain and glare in the sun and provide daily protection against harmful UV rays, by absorbing them. Clothing- Fun and unique decoration that will appeal to some people as they can change, depending on sun exposure. |

| Thermochromic dyes. dyes What is it? It is a smart dye that can be added to plastics and fabrics. | What does it do? Thermochromic dyes change colour in response to changes in temperature |
| --- | --- |
| What are they used for? Novelty fashion The wearer’s body temperature provides heat to change the colour of the jacket. (Stone Island brand) Thermometer strips, baby grows. | Pro’s Clothing- Fun and unique decoration that will appeal to some people as they can change, depending on body temperature. Other- Can be used to visually show if something is too warm or too cold (baby feeding spoons, baby grows, packaging on food/drink products) |